CONSTRUCTION
SITE STATE PLATFORM

Initial Deliverable Proposal

A plug-and-play sensing system for producing a versioned, uncertainty-aware and machine-readable representation of a changing construction site.

Prepared for: Erol
Purpose: Startup concept development and early stakeholder discussions
Status: Working draft v0.1
Date: July 2026

Core proposition

“Give construction teams and future robots an evidence-backed understanding of what is actually on site, how certain that understanding is, and when it was last observed.”

# 1. Document purpose

This document defines an initial deliverable for a proposed construction-robotics startup focused on sensing and site understanding. It is intended to support concept discussions with potential customers, technical collaborators, academic advisers and early investors. It deliberately separates the first demonstrable product from the broader long-term platform vision.

Working product name Site State Capture Package (SSCP): a repeatable site-capture service and software output that combines registered sensor evidence, coverage, change and uncertainty into one usable package.

# 2. Executive summary

Construction sites change continuously, while project information is often fragmented across drawings, BIM models, photographs, surveys, inspection records and informal knowledge. Future construction robots will require an accurate and current representation of the site, but human teams can also benefit immediately from a reliable record of what was observed and where.

The proposed platform connects sensors through standard adapters, records their observations with complete metadata, processes those observations through replaceable software modules, and produces a versioned Site State Model. Rather than claiming a perfect digital twin, the platform explicitly represents uncertainty, coverage, freshness and provenance.

The initial deliverable is a repeatable capture package for one indoor construction area. It provides a registered 3D representation, robot trajectory, spatially linked imagery, change detection between visits, coverage and uncertainty maps, and machine-readable exports. The first commercial value is improved site visibility and as-built/change verification. The same foundation can later support progress tracking, automated inspection, robot navigation and physical task execution.

# 3. Product thesis

Product thesis Build the trusted perception layer for construction robotics: any supported sensor, on any suitable carrier, contributing evidence to the same coherent and continuously improvable site-state representation.

The long-term product is not tied to one mobile robot. The sensing package may be mounted on a wheeled robot, trolley, backpack, tripod, drone or construction vehicle. The robot is an initial acquisition carrier; the durable asset is the software, data model and accumulated site evidence.

# 4. Proposal structure

| Section | Purpose |
| --- | --- |
| Initial deliverable | The concrete output proposed for the first prototype and pilot. |
| Users and use cases | Who receives value and what decisions the package supports. |
| System architecture | How sensors, evidence, processing modules and outputs are separated. |
| Uncertainty model | How confidence, freshness, coverage and provenance are represented. |
| MVP scope | What is included, excluded and intentionally deferred. |
| Validation plan | The demonstration, measures and pilot activities required to test the idea. |
| Roadmap and risks | How the first deliverable may evolve and what could prevent adoption. |

# 5. Problem definition

A construction project typically has a designed state, an actual physical state and a recorded state. These states frequently diverge. Information may be accurate but stale, current but poorly registered, or visually rich but difficult for software and robots to consume.

Site observations are collected by many devices and people but are rarely fused into one consistent coordinate system.

Point clouds and photographs often show what was captured without clearly showing what was missed or how reliable the registration is.

Derived models can lose the evidence and processing history needed to explain why a conclusion was reached.

New perception models are difficult to integrate without rebuilding pipelines or re-capturing historical data.

Robots require structured, current and uncertainty-aware inputs rather than attractive but static visualisations.

# 6. Target users and initial use cases

| User | Immediate value |
| --- | --- |
| Site/project engineer | Review site changes, investigate locations remotely and identify areas requiring follow-up. |
| Quality or commissioning team | Collect evidence and compare observed conditions with planned or prior states. |
| Survey / reality-capture specialist | Extend capture coverage and communicate registration quality and evidence lineage. |
| Robotics integrator | Consume traversability, geometry and confidence outputs for navigation and task planning. |
| Construction subcontractor | Document installed work before it is concealed and reduce repeated site walks. |

RECOMMENDED INITIAL COMMERCIAL WEDGE

Begin with indoor as-built and change verification for commercial fit-out or MEP environments. This creates immediate human value while developing the site representation required by later autonomous systems.

# 7. Initial deliverable: Site State Capture Package

The Site State Capture Package is produced after a sensing mission through a defined project area. It is both a human-reviewable deliverable and a machine-readable dataset. The package must clearly distinguish observation from interpretation.

| Package component | Description |
| --- | --- |
| Registered spatial model | A 3D point cloud or reconstructed surface aligned to a project coordinate frame, with registration quality recorded. |
| Trajectory and localisation record | The carrier path, timestamps, pose estimates and uncertainty throughout the mission. |
| Spatially linked imagery | Images and depth evidence accessible from their acquisition position and orientation. |
| Coverage map | Areas observed, insufficiently observed, occluded or not visited. |
| Change set | Detected differences between the current and a prior capture, with confidence and supporting evidence. |
| Uncertainty view | A spatial view of geometric, temporal, registration and model confidence. |
| Evidence ledger | References to raw data, calibration, software versions, processing activities and derived outputs. |
| Exports and API | Human report plus machine-readable files or services for later applications. |

Acceptance statement for the first prototype The system can capture the same indoor area on two occasions, align both missions to the same coordinate frame, identify meaningful changes, show where capture or interpretation is uncertain, and allow every reported result to be traced to supporting sensor evidence.

# 8. End-to-end workflow

Define the capture area, project coordinate frame and required outputs.

Connect supported sensors through adapters and verify time synchronisation, calibration and health.

Perform a supervised capture mission using a mobile robot or alternative carrier.

Store raw and minimally processed observations in the observation ledger.

Run selected registration, reconstruction, change and semantic processing plug-ins.

Fuse derived claims into the versioned Site State Model without deleting prior interpretations.

Generate the human-reviewable package, API outputs and quality report.

Review flagged uncertainties and optionally re-capture deficient areas.

# 9. Proposed system architecture

The architecture is layered so that hardware, processing models and customer outputs can evolve independently. Each layer has a stable contract and records sufficient metadata for validation and reprocessing.

Figure 1. Proposed layered architecture for the Site State Platform.

## 9.1 Sensor and carrier adapters

An adapter should do more than expose a data stream. It should declare the sensor identity, data types, units, timestamp source, expected accuracy, mounting transform, calibration version, health status and known limitations. This allows the platform to determine whether a sensor configuration is suitable for a requested output.

## 9.2 Observation ledger

The ledger stores evidence and metadata independently from the current world interpretation. Large binary artefacts such as images and point clouds may reside in object storage, while indexed metadata, relationships and processing records may reside in a database. Historical observations must remain available for reprocessing with better algorithms.

## 9.3 Processing plug-ins

Processing modules consume declared inputs and produce typed outputs. Examples include localisation, SLAM, point-cloud registration, reconstruction, segmentation, object detection, BIM alignment and change detection. A plug-in manifest records compatibility, compute requirements, model version, validation results and whether the module is suitable for online or offline use.

## 9.4 Site State Model

The Site State Model contains the current best-supported claims about geometry, entities, relationships and status. Claims remain linked to evidence and can coexist when observations or models disagree. The model is versioned so that users can reconstruct what the system believed at a particular time.

## 9.5 Output adapters

Output adapters translate the internal model into customer and machine-facing formats. Early outputs may include web visualisation, PDF/HTML quality reports, point-cloud packages and JSON APIs. Later adapters may support IFC/BCF workflows, OpenUSD scenes, occupancy maps, navigation costmaps and robot planning scenes.

# 10. Definition of “plug and play”

| Plug-in boundary | Required behaviour |
| --- | --- |
| Sensor plug-in | A new supported sensor can be connected through a documented adapter without changing the world-model core. |
| Model plug-in | A new perception or reconstruction model can be deployed through a manifest-driven interface and compared with prior versions. |
| Carrier independence | The same sensing and processing pipeline operates whether observations come from a robot, trolley, backpack or fixed station. |
| Output plug-in | New customer reports, exports and robotic interfaces can consume the site state without modifying acquisition code. |
| Reprocessing | Previously captured evidence can be reinterpreted with improved models without returning to the site. |

Important design principle ROS 2 may be used extensively at the edge, but the stored data contracts and public interfaces should not require every customer or downstream application to be a ROS system.

# 11. Uncertainty, freshness and provenance

The platform’s differentiating feature should be its ability to communicate the limits of its own knowledge. A single confidence percentage is insufficient; different forms of uncertainty affect different decisions.

| Dimension | Question answered | Example representation |
| --- | --- | --- |
| Geometric | Position, orientation, shape or surface accuracy. | Pose covariance, registration residuals, spatial error estimates. |
| Semantic | Confidence in an object class, state or relationship. | Class probabilities, model disagreement, validation status. |
| Temporal | How current the observation or derived claim is. | Last-seen timestamp, expected change rate, expiry policy. |
| Coverage | Whether the relevant area was actually visible and sufficiently sampled. | Viewpoint density, occlusion, range, image/depth quality. |
| Calibration | Uncertainty introduced by intrinsic/extrinsic calibration. | Calibration version, age, residuals and verification checks. |
| Registration | Confidence that a mission is aligned to the project frame or prior mission. | Control-point residuals, loop-closure quality, drift estimates. |
| Provenance | Ability to explain which evidence and processing generated a claim. | Sensor IDs, evidence links, model versions and processing graph. |

# 12. Core data entities

| Entity | Purpose |
| --- | --- |
| Sensor | Identity, type, capabilities, calibration, health and mounting. |
| Carrier | Robot, trolley, backpack, drone, vehicle or fixed installation. |
| Mission | Capture objective, area, operator, start/end times and configuration. |
| Observation | A timestamped measurement with frame, source and quality metadata. |
| Evidence asset | Image, depth frame, point cloud, trajectory segment or other stored artefact. |
| Processing activity | A specific run of an algorithm or model with parameters and versions. |
| Claim | A derived statement about geometry, an entity, state, relationship or change. |
| Site-state version | A consistent snapshot of accepted and competing claims at a given time. |

# 13. Functional requirements for the MVP

| ID | Requirement |
| --- | --- |
| FR-01 | Ingest synchronised RGB/RGB-D, LiDAR, IMU and odometry data through documented adapters. |
| FR-02 | Record sensor identity, frame transforms, calibration version, health and timing metadata. |
| FR-03 | Create a repeatable mission package containing raw evidence and processing configuration. |
| FR-04 | Register observations within a mission and align repeat missions to one project coordinate frame. |
| FR-05 | Associate images and other evidence with positions in the spatial model. |
| FR-06 | Produce capture coverage and quality indicators. |
| FR-07 | Detect and present spatial changes between two missions. |
| FR-08 | Represent pose, registration, freshness and model uncertainty. |
| FR-09 | Retain provenance from every derived result to source evidence and model version. |
| FR-10 | Expose selected outputs through a simple viewer and machine-readable interface. |

# 14. Non-functional requirements

Modularity: sensors, algorithms and output adapters can be added or replaced independently.

Reproducibility: a processing result can be recreated from recorded evidence, configuration and software versions.

Interoperability: external data contracts are documented and do not unnecessarily expose internal ROS details.

Degraded operation: the system can flag insufficient calibration, timing, localisation or coverage rather than silently producing misleading outputs.

Security and ownership: project data access, export and retention are explicit, with customer ownership expectations defined before a pilot.

Scalability: high-volume binary evidence is separated from indexed metadata and derived state.

Human review: uncertain or conflicting claims can be inspected, accepted, rejected or scheduled for re-capture.

# 15. MVP scope

INCLUDED

One indoor floor or similarly bounded construction area.

One primary sensor configuration: RGB-D or stereo camera, 2D/3D LiDAR, IMU and wheel odometry.

A supervised wheeled robot or trolley-based sensing carrier.

Surveyed targets, fiducials or known control points for project-frame alignment.

Two capture missions of the same area at different times.

Registered point cloud, trajectory, imagery, coverage, change and uncertainty outputs.

A lightweight web or desktop viewer plus an exportable evidence package.

EXPLICIT NON-GOALS

A complete semantic digital twin of every object on site.

Fully autonomous operation across uncontrolled construction environments.

Safety-certified operation around workers without supervision.

Automatic compliance approval or structural defect diagnosis.

A custom mobile base, custom LiDAR or custom compute platform.

Support for every BIM, sensor and file format in the first release.

Heavy manipulation, drilling, lifting or material handling.

# 16. Prototype demonstration

Demonstration scenario Capture the same test construction area on Day 1 and Day 2 after selected objects or temporary structures have changed. Align both sessions, highlight the changes, show unobserved or unreliable regions, and allow each result to be opened back to the supporting images and point-cloud evidence.

| Measure | How it will be assessed |
| --- | --- |
| Repeat registration | Alignment error against independent control points or measured references. |
| Change detection | Precision and recall on a small, manually labelled set of meaningful changes. |
| Coverage transparency | Correct identification of deliberately occluded or unvisited regions. |
| Evidence traceability | Percentage of reported changes or entities linked to reviewable evidence and processing metadata. |
| Mission repeatability | Ability for a non-developer operator to complete a defined capture workflow. |
| Processing latency | Time from mission completion to reviewable package. |
| Robustness | Frequency and nature of failures, manual interventions and unusable data segments. |

# 17. Pilot proposal

A first external pilot should be framed as a jointly evaluated capture and verification exercise, not as deployment of an autonomous construction robot. The pilot should use a bounded area, agreed control references and a small number of decision-relevant questions.

Select one accessible project area and identify the person responsible for interpreting the result.

Agree what changes, installations or progress indicators matter before capture begins.

Perform an initial baseline capture and quality review.

Repeat the capture after meaningful site change.

Review detected changes, missed changes, uncertainty and evidence with the project team.

Document whether the output saved a site visit, improved confidence or identified an issue earlier.

# 18. Initial value proposition

The first value proposition should focus on improved visibility and reliable evidence rather than on replacing construction labour. A customer should understand the deliverable without understanding robotics.

| Customer outcome | Mechanism |
| --- | --- |
| Fewer repeated site walks | Users can inspect spatially registered evidence remotely. |
| Earlier discovery of change | Repeat captures make emerging differences visible before work is concealed. |
| More trustworthy records | Coverage and uncertainty prevent missing areas from being mistaken for completed inspection. |
| Reusable data asset | Historical observations can be reprocessed as better models are added. |
| Foundation for automation | The same site state can later support robot navigation, planning and task verification. |

# 19. Development roadmap

| Phase | Focus | Exit outcome |
| --- | --- | --- |
| Phase 0 - Problem validation | Interview construction teams; collect example capture, rework and verification workflows; identify a pilot site. | A sharply defined user and decision supported by the output. |
| Phase 1 - Evidence rover | Teleoperated capture, time/frame integrity, evidence storage and basic registered viewer. | One complete and repeatable capture mission. |
| Phase 2 - Repeat-site package | Cross-session alignment, change detection, coverage and uncertainty reporting. | The initial Site State Capture Package. |
| Phase 3 - Construction integration | IFC/floor-plan alignment, issue workflows, customer permissions and project exports. | Pilot-ready product for as-built/change verification. |
| Phase 4 - Robot-ready world model | Traversability, dynamic obstacles, task-relevant semantics and planning interfaces. | Site-state service for autonomous applications. |
| Phase 5 - Task modules | Inspection, marking, drilling or manipulation modules using the same platform. | Expansion from sensing into physical construction work. |

# 20. Principal risks and mitigations

| Risk | Initial mitigation |
| --- | --- |
| Too broad to explain or buy | Lead with one repeatable capture package and one decision-relevant use case. |
| Commodity reality capture | Differentiate through uncertainty, provenance, reprocessing, plug-in interfaces and robot-ready outputs. |
| Poor registration accuracy | Use project control points and report measured residuals; do not rely solely on unconstrained SLAM. |
| Changing or inaccessible terrain | Support multiple sensing carriers and supervised capture rather than depending on one autonomous base. |
| Data volume and processing cost | Separate raw object storage from indexed metadata; use tiered retention and selective reprocessing. |
| Model errors presented as fact | Keep observations and claims separate, preserve competing interpretations and require visible confidence. |
| Customer data/security concerns | Define project isolation, local/edge processing options, data ownership and deletion policies early. |
| Insufficient workflow value | Measure saved time, avoided revisits, issue discovery and decision quality during pilots. |

# 21. Key decisions still required

Which first customer segment is easiest to access: commercial MEP/fit-out, general contractor quality teams, survey/reality-capture providers, or robotics integrators?

Which outcome is the first paid deliverable: change verification, as-built capture, progress evidence or robot-navigation input?

What positional accuracy is necessary for that outcome, and how will it be independently verified?

Should the first carrier be a robot, a manual trolley or a detachable sensing mast that supports both?

What level of on-site versus cloud processing will early customers accept?

Which internal representation is minimal enough for an MVP while still preserving versioning, uncertainty and provenance?

# 22. Immediate next actions

| Action | Deliverable |
| --- | --- |
| 1. Customer discovery | Conduct 10-15 structured interviews and obtain examples of current site records, reports and recurring verification problems. |
| 2. Technical spike | Record a small multi-sensor dataset twice in a changing indoor environment and test repeat registration and evidence linking. |
| 3. Data-contract draft | Define minimal schemas for sensor, mission, observation, evidence asset, processing activity and claim. |
| 4. Prototype viewer | Show trajectory, point cloud, images, coverage, changes and uncertainty in a single review interface. |
| 5. Pilot brief | Create a one-page external pilot proposal with required site access, deliverables, evaluation criteria and data handling. |

# 23. Proposed concise description

One-sentence description A plug-and-play sensing and software platform that converts evidence from robots and other sensor carriers into a versioned, uncertainty-aware model of a changing construction site.

Customer-facing description We provide construction teams with a repeatable, evidence-backed view of what is actually on site, what changed, what was not captured and how reliable the result is.

# 24. Conclusion

The proposed initial deliverable is intentionally narrower than a universal digital twin, but it establishes the essential technical and commercial foundation: trustworthy capture, repeat registration, visible uncertainty, complete provenance and replaceable models. Success should be judged by whether a user can make a better project decision from the package, not by the sophistication of the robot or visualisation alone.

If the initial capture package demonstrates repeatability and customer value, the platform can expand into construction-specific semantics, automated verification and robot task support without discarding its original data or architecture.

# Appendix A - MVP output checklist

| Output | Done | Output | Done |
| --- | --- | --- | --- |
| Capture mission record | □ | Sensor/calibration health report | □ |
| Project-frame trajectory | □ | Registered point cloud/surface | □ |
| Spatially linked RGB/depth evidence | □ | Coverage and occlusion view | □ |
| Change set against prior mission | □ | Uncertainty/registration quality | □ |
| Evidence and processing provenance | □ | Human review package | □ |
| Machine-readable export/API | □ |  |  |